TECHNICAL PROPOSAL & SYSTEM SPECIFICATION

Next-Generation Industrial Sort-to-Light System

Enterprise Edge Hardware Architecture & SynaptiX Studio Zero-Code Automation Platform for Fulfillment Centers

| Target Client: | SPX Logistics Vietnam (Shopee Express Fulfillment Operations) |
| --- | --- |
| Pilot Deployment Scope: | 26 Sorting Racks across 5 Primary SOCs (BMT, Vinh, Dien Khanh, Hung Yen, Ha Noi) |
| Document Classification: | Engineering Proposal & Architectural Specification (Ref: SPX-STL-PROP-2026-V1) |
| Solution Platform: | SynaptiX Studio Industrial Automation Platform & GatewayV2 Edge Controller |

■ EXECUTIVE PROPOSITION This proposal delivers an end-to-end industrial automation solution engineered specifically for SPX Logistics Sort-to-Light operations. By integrating our industrial GatewayV2 edge controllers with the SynaptiX Studio visual automation suite, we replace fragile DIY prototype firmware with a robust, zero-code, fail-safe edge architecture capable of sustaining 24/7 high-throughput parcel sorting with zero downtime.

1. Executive Summary & Technical Vision

Operational Context: Modern high-velocity sorting hubs operate under stringent throughput quotas where sorting delays, mis-sorts, or workstation downtime directly inflate cost-per-parcel and degrade end-customer delivery SLAs. During mega-sales campaigns (such as 11.11, 12.12, or Lunar New Year peaks), parcel processing density at regional SOCs surges by over 300%, requiring continuous 24/7 sorting reliability without unexpected line stoppages.

The Maintenance Bottleneck: In conventional Sort-to-Light deployments, edge controllers are frequently built upon ad-hoc microcontroller boards running hard-coded, monolithic firmware. When sorting requirements evolve (such as altering LED timeout windows, switching pick/put logic, adjusting indicator colors for VIP packages, or adding audio beepers), warehouse technicians cannot modify the system on-site. Any minor change necessitates a software developer to rewrite C/C++ firmware, recompile binaries, and physically flash microcontrollers over USB cables across 26 independent sorting racks.

The Solution Paradigm: Our proposed technical solution eliminates this operational liability through an integrated, dual-pillar automation architecture:

1. Industrial Edge Hardware Layer (GatewayV2 + Remote I/O): Decouples cloud/broker connectivity (Wi-Fi 2.4GHz + MQTT Client) from field bus actuation (RS-485 Modbus RTU). A physical hardware separation guarantees that transient Wi-Fi packet drops or broker reconnections will never freeze high-speed LED timing signals.

2. SynaptiX Studio Automation Platform: Eliminates low-level embedded coding entirely. Warehouse automation engineers design, simulate, verify, and wirelessly deploy sorting rules using intuitive Function Block Diagrams (FBD) with 5-stage CRC-16 atomic verification and zero line downtime.

2. End-to-End System Architecture

System Topology: The proposed architecture establishes a resilient 3-tier topology connecting the enterprise WMS/Sorting Server directly down to each individual parcel bin indicator across the 26 pilot racks.

| Tier | System Layer | Technical Function & Communication Role |
| --- | --- | --- |
| Tier 1 | SPX WMS & Sorting Server | Dispatches real-time pick/put routing commands via MQTT broker (spx/sort_to_light/rack_xx/cmd). Ingests rack telemetry, pick timestamps, and diagnostics. |
| Tier 2 | GatewayV2 Edge Controller (Rack Master) | Industrial DIN-rail gateway per rack. Subscribes to broker commands, parses JSON payloads via SynaptiX Rule Engine, and commands local Remote I/O over Modbus RTU (RS-485). |
| Tier 3 | Smart Remote I/O Slave & IC16703 LED Bus | Dedicated real-time microcontroller driving 18 addressable IC16703 LED modules across 3 rack tiers via 74AHCT125 level-shifter with Mean Well 12V 75W PSU. |

2.1 The 1:1 Dual-Module Rack Architecture (Hardware Decoupling)

Architectural Principle: Rather than forcing a single microcontroller to juggle high-level TCP/IP networking, TLS handshakes, JSON deserialization, AND nanosecond-precise LED bit-banging simultaneously, our solution deploys a physically isolated 1:1 master-slave architecture per rack:

GatewayV2 (Edge Master): Manages enterprise Wi-Fi 2.4GHz communication, TLS encryption, MQTT pub/sub queues, and executes the SynaptiX deterministic Rule Engine.

Smart Remote I/O (Actuation Slave): Dedicated slave board isolated on an industrial RS-485 bus (Modbus RTU @ 115,200 baud). Handles the 800 kHz precision waveform generation required by IC16703 / WS2811 LED driver chips without CPU interruption.

■ RELIABILITY ASSURANCE Physical separation of the networking engine from the hardware actuation layer completely eliminates the notorious 'LED flicker and freeze' problem caused by Wi-Fi background interrupts in single-chip designs. Even if the warehouse Wi-Fi fluctuates or reconnects, active LED indicators remain perfectly stable and responsive.

2.2 Standardized Modbus RTU Inter-Module Contract

Data Contract: The GatewayV2 communicates with the Remote I/O slave using an open, deterministic Modbus RTU register mapping over shielded RS-485. This ensures vendor-independence and transparent diagnostic observability:

| Address | Register Type | Data Format | Engineering Function |
| --- | --- | --- | --- |
| 0x0001 - 0x0012 | Holding Register | UINT16 (18 Regs) | Bin 1 to 18 Target State (0=OFF, 1=ON, 2=Blink Fast, 3=Blink Slow) |
| 0x0020 - 0x0032 | Holding Register | UINT16 (RGB565) | Bin 1 to 18 Color Mapping (Packed RGB values per cell) |
| 0x0050 | Holding Register | UINT16 (ms) | Global Auto-Timeout Timer (0 = Latched, 1000..30000ms Auto-Off) |
| 0x0A00 | Holding Register | UINT16 (Bitmask) | Module Diagnostics & Overcurrent / Over-temperature Warning |
| 0x0A20 | Holding Register | UINT16 (Heartbeat) | Safety Watchdog Lease Register (Must refresh every 3000ms) |

3. The Signature Platform: SynaptiX Studio

Zero-Code Industrial Engineering: The cornerstone of our technical solution is SynaptiX Studio—our enterprise industrial automation suite. SynaptiX Studio empowers SPX fulfillment engineering teams to commission, configure, simulate, and supervise Sort-to-Light racks without touching a single line of embedded source code.

3.1 Step 1: Visual Logic Wiring & Dynamic MQTT Ingestion

Visual Canvas Workflow: In SynaptiX Studio, sorting automation logic is constructed visually on an industrial Function Block Diagram (FBD) canvas. Instead of writing custom JSON parsing loops in firmware, engineers simply drag the [MQTT SUB] block and connect it directly to standard IEC 61131-3 functional blocks:

Native MQTT Subscribe Node: Listens to 'spx/sort_to_light/rack_01/cmd' with automatic JSONPath extraction. Extracts target cell ID ($.bin_id), RGB color values ($.color.r, $.color.g, $.color.b), and execution trigger ($.trigger).

Edge Detect Trigger (ON_RISE): Extracts the rising edge of the sorting command to trigger immediate parcel allocation.

Pick Window Timer (TON 5s): Enforces a strict 5000ms operational window. If an operator picks the parcel, the light turns off; if no action occurs within 5 seconds, the timer safely resets the indicators.

Telemetry Publisher (MQTT PUB): Publishes real-time rack heartbeat, active bin ID, and cycle timestamps back to SPX central operations over MQTT telemetry.

Figure: SynaptiX Studio Logic Graph Editor configured for SPX Sort-to-Light Rack #01. Showing visual pipeline: MQTT Subscriber -> Edge Trigger -> 5s TON Timer -> DO0 Actuation -> Telemetry Publisher.

3.2 Step 2: Unified Tag Catalog & Pre-Engineered Blueprints

Centralized Memory Management: SynaptiX Studio centralizes all physical hardware channels and virtual registers into a structured Tag Space. Every physical output, virtual flag, and communication register is mapped directly to standardized Modbus addresses with human-readable aliases and real-time values:

Figure: SynaptiX Studio Unified Tag Catalog (124 Tags). Direct mapping of physical I/Os, virtual flags (VFLAG), and retentive holding registers with live status toggling.

Rapid Template Duplication: To ensure rapid deployment across 26 sorting racks, SynaptiX Studio provides a pre-engineered Blueprint Library. Automation engineers can select the 'SPX Sort-to-Light Standard' blueprint and instantiate a validated, safety-checked configuration onto any rack controller in less than 60 seconds:

Figure: SynaptiX Studio Industrial Blueprint Library. Standardized logic templates (Andon, Logistics Calling, Production Counters) ready for instantaneous multi-rack deployment.

3.3 Step 3: Zero-Downtime Deployment Gate (The 5-Step Safety Pipeline)

Industrial Safety Gate: A catastrophic failure mode in typical warehouse IoT systems occurs when a firmware flash is interrupted midway by a power flicker or Wi-Fi drop, leaving edge controllers bricked and sorting racks dead in the middle of a shift. SynaptiX Studio eliminates this risk through an automated 5-Step Staged Atomic Commit Pipeline:

Figure: SynaptiX Studio 5-Step Deployment Pipeline. Featuring Staged Buffer Upload, CRC-16 Integrity Verification, and Atomic Magic Commit for zero-downtime hot updates.

| Step | Deployment Stage | Verification & Fail-Safe Protection |
| --- | --- | --- |
| Step 1 | Rule & Grammar Validation | SynaptiX compiler inspects topological rule dependencies, cycle detection, and tag range constraints. Rejects invalid logic before transmission. |
| Step 2 | Staged Buffer Upload (0x9010) | Rules are transmitted into an isolated staging buffer in MCU RAM. The active sorting engine continues running the current rules without interruption. |
| Step 3 | CRC-16 Integrity Checksum | The MCU calculates a 16-bit CRC over the staging buffer and compares it against the Studio compiler hash. Any corrupted bit aborts the transaction. |
| Step 4 | Atomic Magic Commit (0xA5A5) | Studio writes the atomic authorization key (0xA5A5) to register 0xA000. In a single scan cycle (sub-1ms), the MCU swaps pointers to activate the new logic. |
| Step 5 | Active Version Confirmation | MCU confirms the active rule checksum and notifies Studio. The rack transition completes with zero sorting downtime. |

4. Edge Hardware Implementation & Field Wiring

Industrial Ruggedization: While SynaptiX Studio provides software intelligence, the physical edge hardware is engineered to withstand demanding warehouse electrical and thermal environments:

4.1 Power Architecture & Safety Margins

Power Budget Analysis: Each rack is powered by an industry-standard Mean Well LRS-75-12 power supply (12V DC, 6.0A, 75W output). The power budget for the 18 LED modules is rigorously calculated:

Module Current Draw: Each IC16703 LED module draws 0.06A @ 12V (0.72W max at 100% full white illumination).

Total Rack LED Consumption: 18 modules × 0.72W = 12.96W total full-load power.

Control Electronics Power: Controller & Gateway idle power: 3.5W.

PSU Utilization: (12.96W + 3.5W) / 75W = 21.9% load factor. Over 78% headroom remains, providing a 5.7× safety margin that prevents power supply overheating and ensures MTBF > 300,000 hours.

4.2 Signal Integrity & Level Shifting Protection

Noise Immunity Measures: Driving high-speed serial LED data lines (800 kHz) across 3 horizontal rack tiers (total harness length ~4.5 meters) introduces substantial capacitive loading and electromagnetic noise from adjacent conveyor motors. Our design implements three strict signal integrity protections:

1. Active 74AHCT125 Level Shifter: Converts 3.3V MCU CMOS outputs to sharp, true 5V TTL logic levels with high drive current (±25mA), ensuring rapid rise times (<10ns) across the harness.

2. Impedance Matching: Each LED data line includes a 33Ω series damping resistor at the transmitter to suppress transmission line ringing and signal overshoot.

3. TVS Surge & ESD Clamping: Data lines and 12V DC power are protected with bidirectional TVS diodes (SMAJ5.0A / SMAJ15A) against electrostatic discharge (ESD) and inductive spikes from warehouse machinery.

5. Industrial Safety, Fail-Safe Mechanisms & Operational SLA

Graceful Degradation: Industrial automation systems in logistics must handle edge exceptions gracefully without operator confusion. Our solution incorporates multi-level fail-safe engines:

5.1 Triple Watchdog Safety Architecture

1. Cloud/Broker Disconnect Watchdog: If Wi-Fi drops or the MQTT broker becomes unreachable for > 5000ms, the GatewayV2 automatically switches rack status to safe mode (pulsing amber tower light), signaling operators that the rack is offline while preventing stale parcel allocations.

2. Inter-Module Lease Watchdog: The Remote I/O slave continuously monitors Modbus register 0x0A20. If GatewayV2 fails to renew the heartbeat within 3000ms, the LED driver automatically blanks all 18 LED modules to eliminate ghost indications.

3. Hardware Task Watchdog: Internal STM32/ESP32 hardware independent watchdog (IWDG) triggers an instant microsecond reset in the rare event of firmware task lockup.

5.2 Maintenance & MTTR Reduction

Modular Field Serviceability: In the event of physical damage to an edge controller (e.g., impact from a hand truck), our modular DIN-rail design enables sub-3-minute replacement (MTTR < 3 min). A technician snaps a replacement module onto the DIN rail, plugs in the pre-keyed terminal blocks, and SynaptiX Studio automatically recognizes the hardware, flashes the rack's golden profile wirelessly, and resumes operations.

6. Value Proposition & Comparative Matrix

Solution Comparison: The table below contrasts the traditional DIY prototype approach against the proposed SynaptiX Industrial Automation Solution:

| Technical Criterion | Traditional DIY Prototype | SynaptiX Industrial Solution |
| --- | --- | --- |
| Control Architecture | Single ESP32 juggling Wi-Fi, MQTT & LED waveforms. Prone to timing freeze. | 1:1 Dual-module (GatewayV2 + Smart Remote I/O over RS-485). 100% decoupled. |
| Logic Customization | Requires embedded C/C++ developers to re-code, re-compile and flash USB. | SynaptiX Studio zero-code FBD editor. Warehouse technicians configure in minutes. |
| Deployment Safety | Full firmware reflash. Risk of bricking rack mid-shift during Wi-Fi glitch. | 5-Step Staged Atomic Commit with CRC-16 staging and sub-millisecond pointer swap. |
| Signal Integrity | Bare 3.3V GPIO driving 5m wires. Signal degradation, flicker and ghost LEDs. | 74AHCT125 true 5V TTL level shifting + 33Ω damping + TVS surge protection. |
| Multi-Rack Scalability | Manual one-by-one USB flashing across 26 racks in 5 SOCs. | One-click deployment via SynaptiX Blueprint Library. Rapid rollout to 100+ racks. |
| Telemetry & Observability | Basic print logs over serial monitor. No remote health monitoring. | Full MQTT telemetry (uptime, pick cycle times, live watch table & diagnostics). |
| Mean Time to Repair (MTTR) | > 2 hours (soldering, breadboard rewiring, manual code reloading). | < 3 minutes (modular DIN-rail hot-swap, pre-keyed industrial screw terminals). |

■ COMMERCIAL & TECHNICAL CONCLUSION By adopting SynaptiX Studio and the GatewayV2 edge architecture, SPX Logistics secures an enterprise-grade automation infrastructure capable of scaling effortlessly from the current 26 pilot racks to hundreds of sorting lines nationwide. We deliver 99.9% sorting uptime, zero-code operational agility, and complete peace of mind during fulfillment peak seasons.